- Multiple Choice (one answer) (40 pts)

We have 20 multiple choice questions, 2 points per question.

A question example is like this:

- In following comparison between RISC and CISC, which one is incorrect?

  - RISC normally has fewer instruction formats

  - RISC normally is more complex to be implemented

  - RISC normally provides fewer addressing modes

  - None

- ~  V.  Short Answer (53 pts in total)

We have 4 short answer questions. Each of the question has some narratives with 3~5 corresponding sub-questions. You should answer the questions based on your knowledge and some simple calculations.

A question example is like this:

Assume we have an SSD with a capacity of 128GB and a page size of 4KB. The SSD adopts the page-level mapping FTL.

a)	If each entry in the address mapping table is 4B size, please calculate the total size of the mapping table.

b)	What are the pros and cons of page-level mapping table? Can you provide some optimizations for it?

VI.	 Discussion (7 pts)

We will have a discussion question. There are a topic and some corresponding requirements, and you need to give a brief answer based on your thinking and understanding of computer architecture.

A question example is like this:

Please discuss at least three design trade-offs when leveraging parallelisms to boost the performance of the processor. Be sure that you provide at least two aspects concerning each design (this is what trade-off means). For example, superscalar processors add more executing units to allow instruction-level parallelism. This design solves the structural hazards in some cases to boost performance, nevertheless, it consumes more resources (chip size, price, power, etc.).